PCM  Connector C1 Blue

Pin #; Wire Color; Circuit #; Function. 1; N/A. 2; N/A. 3; WHT; 1310; EVAP Canister Vent Solenoid Valve. 4; BRN/WHT; 633; Camshaft Position Sensor. 5; N/A. 6; N/A. 7; YEL; 400; VSS Signal return. 8; N/A. 9; PNK/BLK; 1746; VSS Signal return. 10; N/A. 11; DK BLU; 229; VATS fuel enable. 12; LT BLU/BLK; 844; Injector #4 Control. 13; RED/WHT; 2122; Ignition coil 2 control. 14; DK GRN; 49; AC clutch status. 15; BLK/WHT; 845; Injector 5 control. 16; N/A. 17; N/A. 18; YEL/BLK; 846; Injector 6 control. 19; N/A. 20; N/A. 21; PPL; 719; IAT Sensor low. 22; DK BLU; 1225; Transmission range switch signal B. 23; LT GRN/BLK; 1745; Injector 2 control. 24; BLK/WHT; 1704; Ground jumper to other PCM connector. 25; PNK; 1224; Transmission range switch signal A. 26; N/A. 27; RED/BLK; 877; Injector 7control. 28; N/A. 29; N/A. 30; GRY/BLK; 1687; TCS spark retard. 31; DK BLU/WHT; 878; Injector 8 control. 32; ORN/BLK; 434; P/N switch (grounded in park and neutral). 33; BLK; 1744; Injector 1 control. 34; GRY or WHT; 587/687; 3-2 Shift solenoid control. 35; LT GRN; 1222; 1-2 Shift solenoid control. 36; DK BLU; 1936; Fuel level signal to gauge cluster. 37; BRN/WHT; 437; MIL control ( - output to MIL light). 38; N/A. 39; DK GRN/WHT; 459; A/C request. 40; WHTorYEL/BLK; 375/1223; Skipshift light, or 2-3 shift solenoid control. 41; BRN/WHT; 1173; Low oil level light control. 42; LT GRN; 1652; Reverse lockout solenoid (MT only). 43; DK GRN; 335; Low speed cooling fan relay control ( - output). 44; YEL; 573; Crankshaft position sensor signal. 45; DK GRN/WHT; 428; EVAP purge solenoid. 46; N/A. 47; DK GRN/WHT; 762; A/C request. 48; YEL; 492; MAF sensor signal. 49; PPL; 2121; Ignition coil 1 control. 50; N/A. 51; N/A. 52; PPL/WHT; 2128; Ignition coil 8 control. 53; RED; 2127; Ignition coil 7 control. 54; LT BLU/WHT; 2126; Ignition coil 6 control. 55; DK GRN/WHT; 817; VSS signal out. 56; DK GRN; 2125; Ignition coil 5 control. 57; DK GRN/WHT; 2124; Ignition coil 4 control. 58; LT BLU; 2123; Ignition coil 3 control. 59; BRN/WHT; 2130; Reference low even coils. 60; BRN; 2129; Reference low odd coils. 61; RED; 1226; Transmission range switch signal C. 62; BLK; 552; EGR ground. 63; BLK; 407; A/C pressure sensor ground. 64; N/A. 65; N/A. 66; N/A. 67; N/A. 68; N/A. 69; N/A. 70; N/A. 71; PPL; 401; VSS low reference. 72; BLK; 451; Ground. 73; N/A. 74; N/A. 75; GRY; 720; Fuel tank sending unit ground. 76; BLK; 451; Ground. 77; PPL; 420; TCC stop switch, + 12V until brake pedal is depressed. 78; N/A. 79; N/A. 80; N/A.

PCM connector C2 Red

Pin #; Wire color; Circuit #; Function. 1; TAN; 1671; Bank 2 sensor 2 low reference. 2; TAN/WHT; 1669; Bank 1 sensor 2 low reference. 3; TAN; 800; Serial data UART to ALDL pin 9. 4; RED; 631; Camshaft position sensor 12v supply. 5; TAN; 1667; Bank 2 sensor 1 low reference. 6; TAN/WHT; 1653; Bank 1 sensor 1 low reference. 7; RED/BLK; 380; A/C pressure sensor signal. 8; TAN; 472; IAT sensor signal. 9; BRN; 1456; EGR pintle position. 10; PPL; 80; Fuel level sensor. 11; N/A. 12; DK BLU; 417; TPS signal. 13; YEL/BLK; 1227; Trans fluid temp sensor signal. 14; ORN/BLK; 1057`; Trans fluid temp sensor ground. 15; N/A. 16; N/A. 17; N/A. 18; N/A. 19; PNK; 439; Ignition power to 15A fuse. 20; ORN; 420; Battery + to 10A fuse. 21; BLK; 452; Ground. 22; YEL/BLK; 1868; Crankshaft position sensor low reference. 23; BLK; 451; Ground. 24; BLK/WHT; 1704; Jumper to other PCM connector. 25; BRN; 718; ECT sensor ground. 26; ORN/BLK; 469; MAP sensor ground. 27; N/A. 28; DK BLU; 473; High speed cooling fan relay control (- output). 29; N/A. 30; BRN; 436; Air pump relay control. 31; PPL; 421; Air solenoid relay control. 32; TAN/BLK; 422; TCC solenoid control. 33; BRN; 418; TCC (PWM) Solenoid Valve Control. 34; GRY; 435; EGR ground. 35; WHT; 121; Tach signal out. 36; LT BLU/WHT; 1229; Transmission Fluid Pressure Control Solenoid Control Low. 37; DK GRN/WHT; 465; Fuel pump relay control. 38; LT BLU; 697; EGR valve control. 39; RED/BLK; 1228; Transmission Fluid Pressure Control Solenoid Control High. 40; N/A.

Pin #; Wire color; Circuit #; Function. 41; PPL; 1670; Bank 2 sensor 2 Hi. 42; PPL/WHT; 1668; Bank 1 sensor 2 Hi. 43; GRY; 596; TPS 5V reference. 44; N/A. 45; N/A. 46; PPL; 1666; Bank 2 sensor 1 Hi. 47; PPL/WHT; 1665; Bank 1 sensor 1 Hi. 48; LT GRN; 432; MAP sensor signal. 49; YEL; 410; ECT sensor signal. 50; N/A. 51; N/A. 52; DK GRN; 890; Fuel tank pressure sensor signal. 53; N/A. 54; LT GRN/BLK; 444; IAC coil B low control. 55; LT BLU/BLK; 1748; IAC coil A low control. 56; LT BLU/WHT; 1747; IAC coil A high control. 57; LT GRN/WHT; 1749; IAC coil B high control. 58; PPL; 1807; Serial Data to pin 2 of ALDL. 59; N/A. 60; BLK; 451; Ground. 61; ORN; 340; Battery + to same 10A fuse as B+ above. 62; LT GRN; 1867; 12V to crankshaft position sensor. 63; N/A. 64; GRY; 461; MAP 5V reference. 65; GRY; 705; EGR 5V reference. 66; GRY; 474; A/C pressure sensor 5v reference. 67; GRY; 598; Fuel tank level and pressure 5v reference. 68; LT BLU; 1876; Rear knock sensor signal. 69; DK BLU; 496; Front Knock sensor signal. 70; PNK/BLK; 632; Camshaft position sensor low reference. 71; N/A. 72; N/A. 73; N/A. 74; N/A. 75; N/A. 76; BRN; 1174; Oil level sensor signal in. 77; N/A. 78; N/A. 79; N/A. 80; N/A.