| 2006 Hydra-Matic 4T65 Transaxle (M15/MD7/MN7) |  |
| --- | --- |
| Type: | four speed front-wheel-drive, electronically controlled, automatic overdrive transaxle with electronically controlled torque converter clutch. |
| Engine range: | 2.5L - 5.3L |
| Maximum engine torque: | 280 lb-ft (380 Nm) |
| Maximum gearbox torque: | 400 lb-ft (542 Nm) |
| Gear ratios: |  |
| First: | 2.92 |
| Second: | 1.56 |
| Third: | 1 |
| Fourth: | 0.7 |
| Reverse: | 2.38 |
| Final Drive Ratio: ( M15 ) | 2.86, 3.05, 3.29 |
| Final Drive Ratio: ( MN7,MD7 ) | 3.29 |
| Maximum shift speed: | 1-2 7000 rpm |
|  | 2-3 6500 rpm |
|  | 3-4 6000 rpm |
| Maximum validated gross vehicle weight: | 2903 kg (6400 lb) |
| 7-position quadrant: | P, R, N, OD, 3, 2, 1 |
| 5-position quadrant: | P, R, N, D & DSC +/- |
| Case material: | die cast aluminum |
| Shift pattern: | (2) two-way on/off solenoids |
| Shift quality: | variable bleed solenoid |
| Torque converter clutch: | pulse width modulated solenoid control |
| Converter size: | 245mm and 258mm (reference) (diameter of torque converter turbine) |
| Fluid type: | DEXRON® VI |
|  | BOT190M1 - all wheel drive fluid for power take off unit |
| Transmission weight: | 93 kg (205 lb) (M15) (wet) |
|  | 96.6 kg (212 lb) (MN3) (wet) |
|  | 97.2 kg (214 lb) (MN7) (wet) |
|  | 112.5 kg (248 lb) (M76 with power take off (PTO) Unit) (wet) |
| Fluid capacity (approximate): | bottom pan removal: 7L (7.4 qt) |
| Pressure taps available: | line pressure |
| Transfer design: | two-axis design, link chain assembly |
| RPO differences: | M15: 245/258mm torque converter |
|  | MD7 - 245mm China Applications |
|  | MN7: 258mm torque converter and heavy-duty gearbox |
| Assembly sites: | Warren, Mich. (M15, MN7) Shanghai, China (MD7) |
| Applications: | Chevrolet Malibu (M15) |
|  | Chevrolet Malibu Maxx (M15) |
|  | Chevrolet Impala (M15,MN7) |
|  | Chevrolet Monte Carlo (M15,MN7) |
|  | Chevrolet Uplander (M15) |
|  | Buick LaCrosse (M15) |
|  | Buick Lucerne (M15) |
|  | Buick Terraza (M15) |
|  | Buick Rendezvous (M15) |
|  | Pontiac Grand Prix (M15, MN7) |
|  | Pontiac G6 |
|  | Pontiac Montana (M15) |
|  | Saturn Relay (M15) |